Regulations on the Booth Construction of Exhibition

Due to the fact that in the present exhibition sector there are wide differences in strength and qualification among companies providing services for construction of exhibition stands, in order to ensure the standardized construction of exhibition stands for various conventions and exhibitions of National Exhibition and Convention Center, as well as the personal safety and property security of HAH Consulting & Exhibition Co., Ltd. Shanghai, HAH has formulated the following detailed management rules, so as to implement uniform review and supervision over the design, structure, construction and disassembly of participating exhibition stands.

Formality for specially structured exhibition stands:

1．For all specially structured exhibition single stands with an internal height of 4.5m and above, or a roof construction area of more than 50% of the booth exhibition area, and double, multi-storey and outdoor booth (hereinafter referred to as specially structured exhibition stands).The drawings subject to review include:

a.  Overall booth perspective drawing (1front-side and 2 sides)        b.  Ground floor plan

c.  Structural component connection plan                           d.  Cross-sectional booth view,

e.  Anterior booth view                                            f.  Lateral booth view,

g.  Detailed booth material checklist                                h.  Structural drawing

All drawings must be marked with the axis size of beams and posts, as well as specifications and sizes of all structural materials.

2. All construction institutions entering the exhibition hall to start stand construction must provide HAH with copies of their business license certificate。

Management regulations for construction of special design exhibition stand:

- The structure of all specially structured exhibition stands must be reasonably designed, so as to ensure security and safety of the constructed structure. The construction materials shall use non-inflammable or fire retardant materials.

- The designed structural strengths of all exhibition stands shall meet the strength required by the load, during the construction period efforts shall be made to ensure the overall strength, stiffness, stability and local stability of the exhibition stand structure.

- The area of exhibition stand constructed by each construction institution must comply with the area specified in the submitted application. The area of an exhibition stand constructed by construction institutions shall not exceed the leased area, and its projected side line must not extend beyond the leasing border line.

- For exhibition stand structures decorated with glass ornaments, the glass must use safety glass, so as to ensure security in construction and installation, and marked with conspicuous signs in order to prevent glass breaking to cause personal injury or death.

- For exhibition stand using steel structure standing post, the standing post shall use welding-free material with a diameter of over 100mm, its bottom shall be welded with a secure base, with a flange welded onto its upper structure to increase its load bearing area of the standing post, so as to ensure the soundness of exhibition stand structure.

- The ground contact width of the main load-bearing wall in all exhibition stand structures shall not be less than 120mm, so as to ensure the contact area between wall body and the ground.  For walls with a large span of over 6m and steel column framework structures, connecting beams shall be added on the top, with supports fitted in between the columns at the bottom of columns, so as to ensure the overall stiffness and stability of the exhibition stand.

- All exhibitors and construction institutions shall give full consideration to the safety of exhibition stand from design to construction, so as to ensure the soundness of each connecting joints of the exhibition stand and the overall structure of the exhibition stand.

- Double-storey or multi-storey stands must be load-bearing structure of steel structures; the fence and railing of double-storey stands should be made from sturdy, safe material, and can withstand the load specification of the horizontal load.

- Double-storey or multi-storey fence height should not be less than 1.05 m and a minimum width of stair stepping of 0.22 meters and a maximum height of 0.18 meters.

- Construction of double-deck or multiple-deck exhibition stand must be equipped with fire extinguishers that have passed annual inspection.

- Overweight exhibits are not allowed to be placed on the upper floor of the double-storey or multi-storey booths.

- All structural back walls of neighboring exhibition stands must be properly decorated.

- It is strictly prohibited for indoor exhibition stands to adopt totally enclosed type ceilings. Exhibition stand ceilings must not obstruct the fire-fighting facilities on the top of the exhibition hall. It must also be ensured that the ceiling shall have an open space of over 50% of its total area, so as to ensure the fire prevention safety of the exhibition stand.

- The application for all outdoor booth construction area and location of booth must be submitted by the exhibition organizers, once approved by the exhibition centre the application for on-site construction work can then be allowed.

- Outdoor booth design should take full consideration of the safety issues that may be affected by wind, rain and other natural phenomena on the stand. In case of inclement weather conditions during the exhibition, additional inspections should be implemented to ensure exhibition stand safety.

- Any stand structures or exhibition properties (company name, logo, light boxes and posters) are prohibited beyond the boundaries of the booth. Outdoor water proof roof design should be in place, priority should be given to the use of external drainage system.

- Outdoor lighting, sockets and electrical distribution panels, etc. should be of water-resistant type. Outdoor power equipment should implement reliable water-resistant measures and leakage protection device.

- Outdoor exhibition stand shall ensure the strength, stiffness, stability and local stability of the overall structure of exhibition stand, i.e. no variable system is allowed, and there shall be no rather large deformation under load (mainly wind load). The designed structural strength shall meet the requirements of all types of load.

- All construction institutions must not damage all facilities in the exhibition hall or alter its application nature and position. They are not allowed to drive nails, punch holes, paste adhesives, apply colorful paints or post advertisements within the exhibition hall, on exterior floor, or on wall surface.

- When each construction institution is constructing exhibition stand, it shall not set up obstructions in front of the fire-fighting facilities, electric equipment, emergency exits and audience passages, etc. inside the exhibition hall.

- No inflammable and explosive materials shall be used for construction of exhibition stand, and no paint spraying or paint coating shall be carried out inside the exhibition hall.

- When personnel of each construction institution are making mid-air operations, they shall use qualified and safe elevators and operation platform, construction personnel shall tie up safety belt. To protect personal safety, a safety zone shall be set up in its surrounding area, which shall be monitored by dedicated personnel, with conspicuous warning signs set up around the safety zone.

- After the exhibition starts, the construction institution must appoint site safety principal and dedicated personnel to stay on duty at the site, in order to facilitate problem handling when problems crop up.

- For exhibition stands that have safety hazards during construction period, the construction institution must make prompt rectification after receiving “Notice of Rectification”, and provide immediate feedback on the rectification results to HAH Exhibition Site Construction Management Office.

- Construction institutions shall complete the construction within the prescribed time and area, and take care of security work such as fire and burglary prevention. Construction institutions must appoint site principal at the construction site, whose registration and filing shall be done at the time of completing on-site construction work applications.

- Processing tools such as electric saw, electric plane shall not be used for construction inside the exhibition hall, nor shall open fire operations such as electric welding and pneumatic welding be executed.

- The selection of construction materials for exhibition stand must comply with criteria issued by relevant national authoritative departments regarding material usage for temporary buildings, and be implemented in a rational way by taking into consideration the features of the exhibition; the selected materials must conform to national requirements on environment protection and fire-fighting.

- Exhibition stand structure shall not use thin load-bearing structural parts with its pipe wall thickness of less than 0.8mm, nor shall heavily rusted load-bearing structural parts be used.

- HAH Consulting & Exhibition Co. Ltd. will make appropriate adjustments and subsequent confirmation to the “Regulations on the Booth Construction of Exhibition” depending on its specific implementation outcomes, and promptly inform the public.

HAH Consulting & Exhibition Co., Ltd.Shanghai

January 1, 2023